| Checklist: Assessment of results by licensee |  |
| --- | --- |

Criteria for data quality assessment to demonstrate licence compliance

| Criteria - Onsite sampling and analysis |  |
| --- | --- |
| Sampling of discharge/environment: - Was appropriate sample volume collected - Was appropriate sample preservation and storage used - Were samples analysed within appropriate time frame (ISO5667:2003) - Do composite sample and flow time periods correlate | Yes/No |
| Analysis: - Was analysis conducted by suitably qualified & trained staff? - Have the performance characteristic of the procedure been determined? - Has the procedure been assessed for its suitability to the test matrix? - Is the procedure subject to a programme of Analytical Quality control? - Were standard methods (as defined by the licence) used to analyse the sample? - Is all equipment subject to a programme of maintenance and calibration? - Was all equipment within calibration & maintenance dates? | Yes/No |
| Criteria - off site sampling and analysis |  |
| - Has the quality systems and accreditation status of the subcontracted laboratory been assessed to determine that it meets all licence quality requirements in relation to sample collection, analysis and reporting as applicable? | Yes/No |
| Criteria - Final Report |  |
| Does the final report contain the following information - A report title & unique identification number. - Date of sample collection and receipt by laboratory. - Sample type (i.e. grab/composite) composite period (with times) and flow. - Details of the process conditions prevailing at the time of monitoring (where applicable i.e. onsite collection &laboratory analysis). - Method Identification & accreditation status. - Test results and units of measurement. - Form the parameter measurement (e.g. PO4 vs. P, NH4 vs. N). - Any deviations from quality criteria/method during the analysis. - Name, function and signature of senior analyst/chemist who verifies the validity of the result and authorises the test report. - A statement on the estimated uncertainty of measurement. | Yes/No |

Additional Comments:

________________________________			_____________________________

Name & Title						Date