| Checklist: Requirements for External Contractors who conduct aqueoues monitoring to demonstrate compliance with Licence conditions |  |
| --- | --- |

Prior to Monitoring:

| Criteria | Comment |
| --- | --- |
| Has the proposed contractor provided evidence/details of their qualifications, training and experience in carrying out water sampling & emissions monitoring, with particular reference to the scope of work to be completed. |  |
| Have the performance characteristics of the contractors analytical methods been evaluated to determine their suitability for assessing compliance with the licence (e.g. measurement range, LOD, matrix type, precision etc.). |  |
| Has the contractor been made aware of the licence requirements in relation to the monitoring including: - Type of sample required (composite/grab) - Range of parameters to be tested (to ensure suitable containers are brought onsite & used) |  |

On-Site Check of Contractors:

| Criteria | Comment |
| --- | --- |
| Does the contractor have their own monitoring/sample collection checklist/procedures/manual for the parameters being monitored? Are they consistent with the licence requirements? |  |
| Has the contractor requested details of process conditions prevailing on the day of their visit? Have these details been recorded on the sample sheet? |  |
| Has all portable monitoring equipment been calibrated and are certificates available (if applicable)? |  |
| Are any procedures for field/trip blanks measurements being used? |  |
| In the situation were samples are being sent to the analysis laboratory offsite: - Has chain of custody documentation been supplied and used? - Was correct sample collection and where necessary preservation techniques used (Ref: ISO5667: 2003)? |  |
| Has the contractor supplied a copy of the Sample Collection Record Sheet to the licensee which has been signed off by both licensee & contractor? |  |

Note: Health and safety/risk assessment requirements are not addressed as part of this checklist

Monitoring Report:

| Criteria | Comment |
| --- | --- |
| Do the methods used fulfil the requirements of the licence and relevant legislation (i.e. Standard method as defined by the licence, PRTR Regulations)? |  |
| Does the report contain the following information as a minimum? - A report title & unique identification number. | Y N |
| - Date sample was collected & receipt. | Y N |
| - Details on whether the sample was a grab or composite and if a composite the composite period (with times) and flow. | Y N |
| - Was there a replicate/split sample taken? | Y N |
| - Identification of all the methods used and their accreditation status. | Y N |
| - Test results and units of measurement. | Y N |
| - Details of the form the parameter was measured as (PO4 vs. P, NH4 vs. N). (These should be consistent with licence requirements). | Y N |
| - Confirmation that all the quality criteria were met during the analysis. | Y N |
| - Name, function and signature of senior analyst/chemist who verifies the validity of the result and authorises the test report. | Y N |
| - A statement on the estimated uncertainty of measurement. | Y N |
| Is the off-site laboratory accredited for all the analysis which they carried out? |  |

________________________________			_____________________________

Name & Title						Date