| Checklist: Field Sampling and Measurement |  |
| --- | --- |

Possible items required for Sampling: (This checklist is not prescriptive and will vary depending on needs)

- List of the sampling locations/stations – know where you are going

- Maps and/or GPS (with sampling locations pre-entered)

- Field sheets

- Bottles (general chemical, trace metals, microbiological, VOCs, other organics, sediment, other)

- Markers and pens/pencils

- DO meter (checked and calibrated) + spares

- pH meter + buffers

- Thermometer

- Mobile phone

- Bucket & rope (clean)

- Telescopic pole type sampler

- Tape meter or groundwater dipper (for groundwater monitoring)

- Bailer or monitoring/purge pump

- Rods & clamps (for taking microbiological bottles)

- Life jackets

- Boots, waders, rain gear,

- Camera and/or video equipment (with chips, discs, tapes, batteries, etc.)

- Filtration apparatus

- Sample preservative/s

- Fridge or cool boxes and ice packs

- Personal Protective gear (for all possible weather conditions, e.g., survival suits, raincoats, protective footwear, waders, gloves, eye protection, etc.)

- Food and drink – sampler/s may be away in isolated areas for some time

- First aid kit

- Spare batteries

- Contact the laboratory - so that they will expect the samples

- Tape

- Tool for measuring angles at V-notches

- Echo sounder (portable).

- Plastic Bags

- Wash (squirt) bottle

- Volumetric cylinders (plastic – for preparing composite samples)

Check sampling equipment regularly – ensure that it is in good working order – carry spares where appropriate – and repair or replace equipment that becomes damaged or worn out - sampling equipment is relatively inexpensive (compared to the costs involved in laboratory analysis).

Example of Bottles/Preservatives Required for specific analysis

|  | Sample Bottle Types used at the EPA Laboratory, Kilkenny |  |  |
| --- | --- | --- | --- |
|  | Analysis Type | Bottle Type and Volume | Remarks |
| A | General Analysis | 1or 2 litre plastic | Use new 1or 2 litre plastic milk bottles. |
| B | Trace Metals | 100 ml plastic | Add 0.5 ml of conc. HNO3 to the 100 ml sample |
| C | Volatile Organic Compounds - VOCs | 40 ml glass screw-capped vials each equipped with a PTFE-faced rubber septum. | Supplied pre-cleaned and sealed. Do not agitate sample prior to sealing the bottle. Fill the bottle completely to exclude all air. |
| D | Semi Volatile Organic Compounds - Semi Volatiles | 250ml amber glass bottles each equipped with a PTFE-faced rubber septum. | Supplied pre cleaned and sealed. Do not agitate sample prior to sealing the bottle. Fill the bottle to within 1 cm of the top. |
| E | Microbiological – For Drinking Water | 500 ml plastic bottles – each containing 50 mg of sodium thiosulphate | Pre-sterilised – each bottle has a small red label that confirms exposure to radiation. Leave ≈ 2.5 cm air space to facilitate mixing before examination. |
| F | Microbiological – For non-Drinking Water | 250 ml plastic (no sodium thiosulphate added) | Pre-sterilised. Leave ≈ 2.5 cm air space to facilitate mixing before examination. |
| G | Fat Oil & Grease | 1 litre glass | Use a separate 1-litre glass bottle (e.g. a Kilner jar that has been pre-calibrated at 1 litre). Do not sub-divide samples; if duplicate analyses are required then use a separate container for each analysis. |
| H | Phenols | 100 ml amber with 1 ml of 10% of NaOH | Check with the lab re preservatives prior to sampling –Preservatives should be added in the laboratory prior to sampling. |
| J | Cyanide | 100 ml plastic with 1 ml of 10% of NaOH | Check with the lab re preservatives prior to sampling –Preservatives should be added in the laboratory prior to sampling. |

A         B   C    D       E         F         G         H     J

## Example of a Field Sheet

## Name:__________________

## Licence No.______________

Sampling Location Code:

Sampling Location GPS:

Bottle Marking:	 ___________________

Sample No.

Sampling Date:	 ___________________

Sampling Time:	___________________

Duplicate Sample

Split Sample

Sampled by: 	___________________

Contact Person: ___________________

Temperature:	___________________°C

# Composite Sample

Start		Finish

Date		_________	________

Time		_________     _________

Flow Meter	_________	_________ Reading

Units		_________	_________

# Grab Sample

# V-Notch

Angle: _________°	   Head: _________cm

# Weir/Flume

Width: _________cm	   Head: _________cm

# Meter Reading

_________	   Units: _________

Comments:

Continue overleaf if necessary

Sample Bottles Required:	Chemical 	Metals fix 	VOC’s  SVOC’s

Tests Required: 		pH, BOD, COD, Kone, Suspended Solids etc...

## Example of a Chain of Custody Document

- Origin of Samples

## Samples from: _________________________________________________________________

_____________________________________________________________________________ Sampling Date: __________________________ Sampled by: ___________________________

(Signature of Sampler)

| Laboratory no.* |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bottle Marking |  |  |  |  |
| Sample Location and Description |  |  |  |  |
| Sampling Time |  |  |  |  |
| Comments |  |  |  |  |
| Temperature °C DO % Satn. |  |  |  |  |

2. Courier (This section to be completed where the courier and sampler are different persons)

The samples as described above were passed from the sampler to the courier

at ___________ hours on ___/___/______.

Signed: ________________				______________________

(Sampler)						(Courier)

3. Receiving Laboratory

Received at: _________________________________________ Laboratory

at __________________ hours on ___/__/______.

Signed: _____________________		           _________________________

(Sampler or Courier)			            (For Receiving Laboratory)

The original of this sheet is to be retained in the sampling laboratory.  Use the back of this sheet for additional comments, sketched etc.

*The Laboratory Number id to be assigned on receipt at the sampling laboratory